rewrite it in terms of byte numbers instead of packet number

Yes — the lecture already treats TCP in terms of byte numbers, where a segment is identified by the number of its first byte, and an ACK number is the next expected byte.[1]
So the cleanest rewrite is to avoid packet labels like p1000 and instead talk directly about the segment whose first byte is 1000 or byte 1000 being the first unacknowledged byte.[2][1]

Byte-number rewrite

Q8 TCP (10 points) Assume a TCP sender has an MSS of 100 B and a window size of 400 B. There is no congestion control, so the window size is constant. The sender has just received ACK 1000, meaning the receiver has received all bytes up to byte 999 and is now expecting byte 1000. The timeout interval is constant and is denoted by RTO. Unless otherwise stated, all transmitted segments experience an RTT of R. Assume segments are never fragmented or reordered, and that losses occur only when explicitly stated. Each segment carries 100 bytes and is identified by the sequence number equal to the number of its first byte. Thus, the sender may transmit segments starting at bytes 1000, 1100, 1200, and so on. Assume TCP keeps a single timer for the segment containing the first unacknowledged byte, and when the first unacknowledged byte advances, the timer becomes associated with the new left edge of the window. All subparts are independent.[2][1]

Q8.1 (1 point) Immediately after the sender processes ACK 1000, which byte is the first unacknowledged byte, and which segment is the timer associated with?[1][2]

Solution: The first unacknowledged byte is 1000, so the timer is associated with the segment whose first byte is 1000.[2][1]

Q8.2 (2 points) At the moment the sender transmits the segment whose first byte is 1600, what is the highest ACK number the sender must already have received?[1][2]

Solution: 1300. With a 400-byte window, transmitting the segment starting at byte 1600 means the send window is [1300,1700), so the first unacknowledged byte must be 1300, which means the sender must already have received ACK 1300.[2][1]

Q8.3 (4 points) Assume that the only lost segment is the one whose first byte is 1400.[2]

(a) How does the sender detect the loss?[1][2]

- Timeout[2]

- 3 duplicate ACKs[1][2]

- The sender does not detect the loss[2]

Solution: 3 duplicate ACKs. Since later in-window segments can still arrive, the receiver keeps sending duplicate cumulative ACKs for byte 1400, and after three duplicates the sender retransmits the missing segment.[1][2]

(b) After the retransmitted segment starting at byte 1400 is received successfully, what ACK number does the receiver send?[1][2]

Solution: 1800. The receiver can now cumulatively acknowledge all bytes through 1799, so it sends ACK 1800, meaning byte 1800 is the next expected byte.[2][1]

Q8.4 (3 points) Assume every segment whose first byte is 1400 or larger is dropped. When does the timer expire?[1][2]

Solution: t1300+R+RTO, equivalently t1700+RTO. Once ACK 1300 is received, byte 1400 becomes the first unacknowledged byte, so the single timer is now associated with the segment starting at byte 1400 and expires RTO later.[2][1]

Wording changes

The important lecture-consistent shift is to say “ACK 1000” rather than “highest ACK received is for byte 1000,” because lecture notation defines the ACK field as the next expected byte.[1][2]
Likewise, it is clearer to ask for the highest ACK number received rather than the “highest ACKed byte,” since ACK 1800 means bytes through 1799 were received contiguously.[2][1]

Even cleaner version

If you want it maximally byte-oriented, Q8.1 can ask only for the first unacknowledged byte, and Q8.4 can ask “when does the timer for byte 1400 start, and when does it expire?”.[1][2]
That matches the lecture’s sliding-window language, where the window begins at the first unacknowledged byte and the retransmission logic tracks the left edge of the window.[1]

⁂

- Lecture-11-TCP-Implementation.pdf

- cs168-sp24-midterm-sols.pdf