# CS 168	Routing II

Spring 2026	Discussion 4

# Link-State Routing

3

R7

A

2

R1

1

19

R2 	8	 R3

1

R6

R9

1

R8

R4 	4	 R5

2

B

3

3

1

1

For this problem, assume the network is running a link-state routing protocol, minimizing total route latency. The following questions indicate events that happen consecutively.

1.1

After convergence, what route does R7 think its packet will take to Host B? R7 → R8 → R9 → R5 → B

EVENT: Ge R8-to-R9 link goes down.

1.2

R8 and R9 have recomputed their routes but have not yet sent updates to other routers. What route does

R7 think its packet will take to Host B? Same, R7 → R8 → R9 → R5 → B

1.3

1.4

What route does it actually take? R7 → R8 → R4 → R5 → B

Assume all nodes are now aware of the new network state and have recomputed their routes. What route does a packet take from R3 to Host A?

R3 → R7 → R6 → R2 → R1 → A

- Routing II

EVENT: Ge cost of the R1-to-R2 link increases to 100.

1.5

R2 and R1 recompute their routes but have not yet sent updates to other routers. What route does R2

think its packet will take to Host A? R2 → R6 → R7 → R3 → R1 → A

1.6

1.7

What route does it actually take? R2 → R6 → R2 (loop)

Which additional routers must receive the routing updates and recompute their routes for all routers to be able to successfully send packets to Host A?

R3, R6, R7

1.8

All routers except R3 have received the routing updates and recomputed their routes. Which routers can successfully send packets to Host A?

None of the routers (except R1)

# More L3 Link State

A

R1 	8	 R2

2

3

4

4

R3 	2	 R4 	3	 R5

R6

2

3

4

4

B

1			1

R7

R8

8

2.1

After convergence, what is the path cost from A to B, and what are all the possible paths with this cost? The cost is 15. All paths are A → R3 → [R1, R4, R7] → R5 → [R2, R8] → R6 → B.

2.2

Suppose that a control message (a message used by the routing algorithm) takes 1 second to propagate along a link, regardless of link cost. What individual link failure inside the network would cause the longest delay to reconvergence, and what is that delay?

Routing II	3

A link R3-to-R4 failure would cause a 3-second delay. The last node that learns about the failure is node R6, and the update must traverse through at least 3 links to get to R6, which will take 3 seconds. Any other link failure within the network can propagate to every other node in less than 3 seconds. Host links could also take 3 seconds to propagate.

2.3

Suppose you have the ability to take down individual nodes. Which nodes would you take down in order to partition the network? If you can’t partition the network, which nodes would you take down to increase path costs from A to B maximally?

In each part, suppose you can take down:

  - A single node (excluding R3 and R6).

R5. Can’t partition, and removing R5 increases path costs the most (by 1).

  - Two nodes (excluding R3 and R6).

R2 and R8. Removing them will partition R6 from the network.

2.4

Which single link’s cost (if any) should you double in order to increase the path cost from A to B?

Doubling the cost of A-R3 or R6-B will increase the total path cost by 1. It’s not possible with any other link, since there is always an alternate, cheaper path (see part 1).

# IP Addressing

Suppose berkeley.edu is the Provider AS for EECS, CS, Math, and Chemistry, and needs to assign IPv4 addresses to them.

Assume that the CIDR (Classless InterDomain Routing) addressing scheme is used, and that berkeley.edu

has the 123.100.0.0/16 prefix reserved.

EECS

CS

UC Berkeley

Math

Chemistry

123.100.0.0/16

123.100.128.0/18

123.100.0.0/18

3.1

Which addresses are included in the Math department’s prefix? How many addresses are in this range?

- Routing II

Math’s address prefix, in binary (with the prefix bolded), is:

01111011 . 01100100 . 10 000000 . 00000000

The range of allowed addresses is thus:

01111011 . 01100100 . 10 000000 . 00000000

…

01111011 . 01100100 . 10 111111 . 11111111

which in decimal is [123.100.128.0, 123.100.191.255].

If Math’s prefix is 18 bits long, and IPv4 addresses are 32 bits long, then there are 32 - 18 = 14 bits available to uniquely identify hosts, so this prefix contains 2¹⁴ addresses.

3.2

123.100.192.0/18 is reserved for EECS and CS. Assign equal halves of this address space to the two departments.

This address space has the prefix: 01111011 . 01100100 . 11 000000 . 00000000

To divide this /18 prefix in two, we can assign one /19 prefix to each department:

EECS: 01111011 . 01100100 . 11 0 00000 . 00000000 123.100.192.0/19

CS: 01111011 . 01100100 . 11 1 00000 . 00000000 123.100.224.0/19 (or vice versa)

3.3

You want to start a new department, Floriology, and assign it an unused address range. You foresee that no more than 50 people will enroll. Assuming one address per person, what prefix would you assign to it?

A 64-address prefix will hold enough addresses for Floriology, which requires 6 bits for hosts. Thus, we need a (32 - 6) = /26 prefix.

We have to assign a prefix that is currently unclaimed. If EECS/CS claimed 123.100.192.0/18, Math claimed 123.100.128.0/18, and Chemistry claimed 123.100.0.0/18, then only the 123.100.64.0/18 prefix is unclaimed.

Any /26 prefix in 123.100.64.0/18 will work, such as 123.100.64.0/26.