Lecture 11 - TCP Exercises ANS

# TCP in Action

Consider a sender sending 1000 B of data to a receiver over TCP. The sender sends packets of 100B, the window size is 300B, and the ISN is 99 (so D100 is the first packet sent, then D200, and so on).

Remember, TCP uses a sliding window and retransmits the packet containing the next expected byte on a timeout or, if TCP Fast Retransmit is enabled, when three duplicate acks are received. Assume here that TCP Fast Retransmit is not enabled.

The link is flaky! The initial transmission of packets D200 and D700 get dropped.

1.1 Fill in the below table with all packets sent by the sender until the receiver has received all packets and the sender knows that. For simplicity, assume that packets (data and ACKs) arrive in order. You may or may not need to fill in all lines.

|  | Packet Data Offset | Sent on timeout | Dropped? | Cumulative ACK |
| --- | --- | --- | --- | --- |
| 1 | D100 |  |  | A200 |
| 2 | D200 |  | X |  |
| 3 | D300 |  |  | A200 |
| 4 | D400 |  |  | A200 |
| 5 | D200 | X |  | A500 |
| 6 | D500 |  |  | A600 |
| 7 | D600 |  |  | A700 |
| 8 | D700 |  | X |  |
| 9 | D800 |  |  | A700 |
| 10 | D900 |  |  | A700 |
| 11 | D700 | X |  | A1000 |
| 12 | D1000 |  |  | A1100 |
| 13 |  |  |  |  |
| 14 |  |  |  |  |

2 	Transmission Control Protocol

1.2 If the RTT of the link is 10ms and the timeout is initially 3 seconds, what is the total time needed for the receiver to receive all packets and for the sender to know that?

Assume small packets (negligible transmission delay) and negligible processing time, and that the estimates that go into the Retransmission Timeout (RTO) remain constant during the events below.

2*RTO + 5 RTT = 6.05 s

1*RTT for 1, 1*RTO before 5, 2*RTT for 5-7, 1*RTO before 11, 2*RTT for 11-12

Note that we have 2*RTT for 5-7 – not 3*RTT. Because D500 and D600 are sent at practically the same time, it takes  1 RTT from when we send D500 until we receive A700.

# TCP Calculations

In this question, quantities will be measured in bytes unless explicitly mentioned otherwise.

2.1 Suppose two hosts are about to open a TCP connection. The TCP headers used in the communication are only 20 bytes long and regular (no-options) IPv4 is being used for Layer 3. If the MTU of the link is 1260 bytes, what is the MSS?

1220 bytes = 1260 bytes - 20 bytes (TCP) - 20 bytes (IP)

2.2 When this connection starts, the sender starts with an ISN 19. The initial window for the sender is set to 10 packets. Given the previously calculated MSS, what ACK does the sender receive as part of the TCP handshake? After that, what is the first and last ACK the sender receives for this initial window? (Assume no packets were lost or reordered).

The ACK as part of the TCP handshake will be 20, because the receiver is ACKing having received the ISN. After that, the first ACK for real data sent by the sender will be 1240, which is the MSS (1220, calculated in the last problem) + the current sequence number, which is 20. The last ACK will be 12220, which is the next sequence number after all data is sent, 12200 + ISN = 12200 + 20 = 12220.

2.3 TCP senders compute their retransmission timeout (Estimated_Timeout, RTO) using exponentiallyweighted moving averages of measured round-trip times (Estimated_RTT) and the deviations of those RTT measurements (Estimated_Deviation). Every time a new ACK is received, the sampled RTT of that specific ACK (Sampled_RTT) is used to perform an online update, by making the following three calculations in order (where 𝛼, 𝛽, and 𝜅 are fixed TCP parameters):

Estimated_Deviation = (1−𝛽)×Estimated_Deviation+𝛽 ×|Estimated_RTT−Sample_RTT|

Estimated_RTT = (1−𝛼)×Estimated_RTT+𝛼×Sampled_RTT

Estimated_Timeout (RTO) = Estimated_RTT+𝜅×Estimated_Deviation

Now, suppose that we are a sender and have received a new ACK, and want to perform one step of RTO estimation. Let’s do each of the three calculations above in the following suparts. Assume that connections have been open and that there are no dropped packets.

(a) The sender receives the current ACK and proceeds to update its various estimations. The time from when the packet was sent to when the ACK was received was 10msec. If the previous Transmission Control Protocol 	3

Estimated_Deviation was 50msec, and the previous Estimated_RTT was 70msec, what is the new Estimated_Deviation (using β = 0.5)?

Estimated_Deviation = 0.5×Estimated_Deviation +0.5×|Estimated_RTT−Sample_RTT|

= 0.5×50 msec	+0.5×(70 msec	−10 msec)

## = 55 msec

- Still using the previous Estimated_RTT of 70msec, What will the new Estimated_RTT be (using α = 0.5)?

Estimated_RTT = 0.5×70 msec+0.5×10msec = 40 msec

- Based on the previous two questions, what will the ETO be now (using 𝜅=4)?

## ETO = 40 msec+4×55 msec = 260 msec

2.4 What is the maximum theoretical rate of data transfer for this window size if the Estimated_RTT is what was previously estimated?

Window_Size (bytes) = Window_Size (Packets) × 1220 bytes = 12200 bytes

Bandwidth = 4102𝑚200sec𝐵 = 305000 sec𝐵 = 305 KBps

2.5 Assume 40msec is the Estimated_RTT. If the lowest bandwidth across this connection is 76.25 MBps, what is the smallest window that optimizes the question?

76.25sec×106𝐵 × 40 msec = 3.05 MB