# Lecture 11: TCP Implementation Quiz ANS

1. In real TCP, what is the primary unit used for sequence numbers and ACKs?

- A. Packets

- B. Bytes

- C. Segments

- D. Frames

ANS:

2. What does the sequence number field in a TCP header represent for a data segment?

- A. The number of the last byte carried in the segment

- B. The count of bytes transmitted so far on the connection

- C. The number of the first byte of data in the segment

- D. The total number of segments sent so far

ANS:

3. Suppose a TCP sender transmits a segment with sequence number j carrying B bytes. If all prior data has been received in order, what ACK value will the receiver send after correctly receiving this segment?

- A. j

- B. j + B − 1

- C. j + B

- D. j + 2B

ANS:

4. In normal operation with one packet in flight, no loss, and fixed segment size B, how are the ACK number and next sequence number at the sender related?

- A. ACK number is always smaller than the next sequence number

- B. ACK number equals the next sequence number

- C. ACK number is always larger than the next sequence number

- D. There is no simple relationship

ANS:

5. Why does TCP choose a random Initial Sequence Number (ISN) for each new connection?

- A. To minimize header overhead

- B. To ensure that segments are aligned to MSS boundaries

- C. To improve security and avoid confusion with stale segments from old connections

- D. To simplify retransmission timeout estimation

ANS:

6. If all bytes up to and including ISN+219 have been received correctly, which ACK value will the receiver send next?

- A. ISN + 219

- B. ISN + 220

- C. ISN + 218

- D. ISN + 221

ANS:

7. Which combination of TCP header fields is primarily responsible for implementing reliability?

A. Source port and destination port

B. Sequence number, acknowledgment number, and checksum

C. Advertised window and header length

D. Flags and urgent pointer

ANS:

8. If an ACK for byte 100 is lost but later an ACK for 120 arrives. What can Host A infer from the ACK=120?

- A. Only bytes 100–119 were received; 92–99 must be retransmitted

- B. All bytes up to 119 have been received, including 92–99 and 100–119

- C. Only the segment starting at 120 was received

- D. The connection should be reset

ANS:

9. TCP connections are full-duplex. What does this mean in practice?

- A. Data can only flow in one direction at a time, but the direction can switch

- B. Both hosts can send and receive data simultaneously over the same connection

- C. The connection uses separate ports for sending and receiving

- D. Each segment can carry data or acknowledgments, but not both

ANS:

10. In a full-duplex TCP connection, how many independent sequence-number spaces are used?

- A. One, shared by both directions

- B. Two, one for each direction A→B and B→A

- C. Four, one per host and per port

- D. One per segment

ANS:

11. What is the main purpose of the three-way handshake in TCP connection setup?

- A. To exchange congestion window sizes

- B. To synchronize IP addresses between client and server

- C. To allow each host to learn the other’s ISN and establish ISNs (initial sequence-numbers)

- D. To negotiate MTU along the path

ANS:

12. During normal TCP connection termination, what does a FIN flag indicate?

- A. The sender will neither send nor receive any more data

- B. The sender is done sending but will continue to receive data

- C. The sender is aborting the connection immediately

- D. The sender is requesting a higher advertised window

ANS:

13. How does the behavior of a TCP RST segment differ from a FIN segment?

- A. RST must be acknowledged while FIN does not

- B. RST closes only one direction; FIN closes both

- C. RST immediately aborts the connection without requiring an ACK, discarding in-flight data

- D. RST is used only during connection setup

ANS:

14. In the TCP state machine, which state represents a connection that is fully open and exchanging data?

- A. SYN_SENT

- B. LISTEN

- C. ESTABLISHED

- D. TIME_WAIT

ANS:

15. In TCP's byte-based sliding window, a sender maintains a single retransmission timer rather than one timer per segment. Under what condition does the sender retransmit without waiting for the timer to expire?

A. When any ACK is received for a byte inside the current window

B. When three duplicate ACKs for the same byte are received

C. When the advertised window drops to zero

D. When the congestion window exceeds the receiver's advertised window

ANS:

16. What is "piggybacking" in the context of TCP?

- A. Sending data without any ACKs

- B. Delaying data segments until the congestion window grows

- C. Combining ACK information with outgoing data in the same segment

- D. Using multiple connections over the same port

ANS:

17. Why are SYN-ACKs always piggybacked in TCP?

- A. Because data must always be sent with SYN segments

- B. Because the OS, not the application, performs the handshake and can easily combine SYN and ACK in one control segment

- C. To reduce the number of sequence-number spaces needed

- D. To avoid triggering congestion control

ANS:

18. In the TCP sliding-window scheme (measured in bytes), what does the window [first_unacked, first_unacked + W) represent?

- A. All bytes that have been successfully sent and acknowledged

- B. The set of bytes that are in flight but not be acknowledged yet

- C. The total buffer space at the receiver

- D. The advertised congestion window only

ANS:

19. Which events cause the sliding window to move (slide) forward?

- A. Any ACK that is received for any byte within the window

- B. ACKs that acknowledge the leftmost unacked bytes, thereby advancing the first unacked byte

- C. Timer expirations only

- D. Arrival of out-of-order data at the receiver

ANS: B

20. TCP sets its effective send window W as the minimum of two quantities. Which two?

- A. Congestion window and MSS

- B. Receiver advertised window and MSS

- C. min(RWND, CWND), Congestion window and receiver advertised window

- D. RTT estimate and MSS

ANS: